Scenario Analyzer

0.0.5 Exercises

May 19, 2010

EXAMPLE OPERATIONS

- Calculate drawdown and create map

Run ScenarioAnalyzer.exe by double clicking on the executable.

Add a New Map by right-clicking in left pane of the window.

Name the map Drawdown and click Ok.

Add a New Contour Layer by right-clicking on the Drawdown Map item in the left window.

Name the contour data series Pump, set the data type to Heads Map, select the heads file 
..\ PumpingResults\SWRSample08.hds, select layer 1, stress period 10, and timestep 1.

The layer you added will appear on the map preview.

Add another Heads Map layer. Name this one No Pump and select the heads file in the NonPumpingResults folder.

Add a Dataset Calculator layer named Drawdown. Choose red in the Line Series Color box. For the expression, input [No Pump] – [Pump]. You can input a layer name in the expression box by double-clicking on the layer name in the list.

The Drawdown map will appear on your map.

Select Properties in the Pump context menu. This will show the dialog box that you used before to configure this layer.

In the Line Series Color box, select No Line Series. Click OK and repeat this process with the No Pump layer.

You now have a map that shows the drawdown for stress period 10, timestep 1 in this model.

- Create a color flood of heads for the pumping simulation for a subset of the model domain

Add a New Map by right-clicking in left pane of the window.

Deselect Show Basemap in the map dialog. Name the map Pumping Heads.

Select the Extent tab and change the Extent to Manual and the maximum value of the y range to 50.

Add a New Color Flood Layer to your Pumping Heads map.

Create a Heads Map named Heads. Select the heads file in the PumpingResults folder and layer 3, stress period 10, and timestep 1.

Add a New Contour Layer to your Pumping Heads map.

Name this layer Head Contour and choose the same data type, heads file, layer, stress period, and timestep as the previous layer. Choose a red line for Line Series Color (if you’re feeling really adventurous, you may even choose a green line).

The Head Contour layer is not visible on your map preview. This is because it is being obscured by the Heads color-flood layer.

Open the Properties of the Pumping Heads map.

On the Data Series tab, select the Head Contour map and click the up arrow on the right side of the dialog.

The Head Contour map will move up one position in the list.

Now, you can see the Head Contour layer in the map preview.

In the File menu, select Save As… and save your document to a convenient location.

- Extract timeseries of water levels at a point

Close and reopen the application. Select Open… in the File menu.

Open the document that you just saved.

Your document is exactly as you left it before you closed the application.

Add a New Graph to your document.

Name this graph Heads and pumping wells.

Add a New Data Series to the graph.

Name the data series PW-01, chose Head At Point for the data type, select the heads file from the PumpingResults folder, and choose column 9, row 25, layer 3.

Repeat this for the second well. Name this one PW-02, select a different line color, and choose column 25, row 33, layer 3.

Export a CSV of the PW-01 time series.

Save the CSV file to a convenient location.

Add another New Data Series to your latest graph.

Make this an Observed Series (SMP file). Select the PW-01_Heads.smp file in the SWRSample08 folder. Select No Line Series and a Point Series Color of black. Name the series PW-01 Obs.

You now have a graph that shows simulated heads at two wells and also shows observed heads at one of those wells.

- Extract SWR1 cell-by-cell flows for sub-basins

Add another New Graph to your document. Name this SWR1 Exchanges.

Add a New Data Series to this graph. Call this series Pumping, select the CBB file from the PumpingResults folder, choose the SWR LEAKAGE dataset, and set the layer range from 1 to 3.

Click the Add New Group button and browse to the SWRSample08_Basins.csv file in the GroupFiles folder.

This file is a simple text file that contains the cell indices for all basins in this simulation. The following image shows these basins.

Select Basin02 in the Groups box.

The graph preview shows the sum of SWR leakage for all cells in the selected basin.

Add another Data Series. Name this one No Pumping and select the CBB file in the NonPumpingResults folder. Select the same dataset, layer range, and group as for the previous data series.

Add a Dataset Calculator series. For the expression, input [No Pumping] – [Pumping]. Name this series SWR1 Flux Difference. Choose a different line series color than the other series.

The graph now shows the time series data of the pumping and non-pumping simulations, as well as the difference of these simulations, for all cells in basin 2.

- Add a new graph and evaluate the difference in SWR1 exchanges for basin 4

Repeat the previous exercise. This time, create a graph for basin 4. The zero line can be created by adding a Dataset Calculator series and typing the number 0 for the expression.

- Create a Table

Add a New Table to your document.

Name this table Sampled Heads.

Add a New Data Series to the table.

Name this series Sampled Times. Browse to the the Sample01.smp file.

The only series in this file is named Sample01. In a file with multiple series, you could select the appropriate time series from the Series list.

A preview of the table will appear in the preview pane.

Add a New Data Series and select the heads file from the PumpingResults folder. Select column 9, row 31, and layer 3.

The new data series will appear as a third row in your table.

Repeat this process for the second well. This well is located in column 25, row 33, and layer 3.

This well will appear as a fourth column in your table.

Add a Cell-By-Cell File Point Series for the CBB file in the PumpingResults folder. Choose the WELLS dataset. The well is located in column 9, row 41, layer 3.

This series will appear as a fifth column in your table. You may wish to resize the window to be able to see the table more clearly.

Choose Save from the File menu to save your document again.

- Export to the document to a PDF report

Right-click on an empty area of the left pane and select Export to PDF.

Select a convenient location for the PDF and save it.

A confirmation dialog will inform you that the report has been saved to the path you selected.